H316 Simulator Usage

09-Oct-2013

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2013, written by Robert M Supnik

Copyright (c) 1993-2013, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	H316/H516 Features	3

2.1	CPU	4

2.2	Programmed I/O Devices	5

2.2.1	316/516-50 Paper Tape Reader (PTR)	5

2.2.2	316/516-52 Paper Tape Punch (PTP)	6

2.2.3	316/516-33 Console Teletype (TTY)	7

2.2.4	316/516-12 Real Time Clock (CLK)	8

2.3	316/516 Line Printer (LPT)	8

2.4	4400 Fixed Head Disk (FHD)	9

2.5	4100 7-track Magnetic Tape (MT)	10

2.6	4623/4651/4720 Disk Packs (DP)	11

3	Symbolic Display and Input	13

This memorandum documents the Honeywell H316/H516 simulator.

# Simulator Files

The H316 requires the following files:

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/h316/	h316_defs.h

h316_cpu.c

h316_fhd.c

h316_lp.c

h316_mt.c

h316_dp.c

h316_stddev.c

h316_sys.c

# H316/H516 Features

The Honeywell 316/516 simulator is configured as follows:

device names	simulates

CPU			H316/H516 CPU with 16/32KW memory

PTR			316/516-50 paper tape reader

PTP			316/516-52 paper tape punch

TTY			316/516-33 console terminal

CLK			316/516-12 real time clock

LPT			316/516 line printer

FHD			4400 fixed head disk

DP			4623/4653/4720 disk pack controller with eight drives

MT			4100 seven track magnetic tape with four drives

The H316/H516 simulator implements several unique stop conditions:

- Decode of an undefined instruction, and STOP_INST is set

- Reference to an undefined I/O device, and STOP_DEV is set

- More than INDMAX indirect references are detected during memory reference address decoding

- DMA/DMC direction does not agree with I/O device operation

- A write operation is initiated on a write locked magnetic tape unit (hangs the real system)

- A disk write overruns the specified record size (destroys the rest of the track on the real system)

- A disk track has an illegal format

The LOAD and DUMP commands are not implemented.

## CPU

CPU options include choice of instruction set, memory size, DMC option, and number of DMA channels.

SET CPU HSA				high speed arithmetic instructions

SET CPU NOHSA			no high speed arithmetic instructions

SET CPU 4K				set memory size = 4K

SET CPU 8K				set memory size = 8K

SET CPU 12K				set memory size = 12K

SET CPU 16K				set memory size = 16K

SET CPU 24K				set memory size = 24K

SET CPU 32K				set memory size = 32K

SET CPU DMC				enable DMC option

SET CPU NODMC			disable DMC option

SET CPU DMA=n			set number of DMA channels to n (0-4)

If memory size is being reduced, and the memory being truncated contains non-zero data, the simulator asks for confirmation.  Data in the truncated portion of memory is lost.  Initial memory size is 32K.  By default, the HSA and DMC options are enabled, and four DMA channels are configured.

The CPU includes special show commands to display the state of the DMA channels:

SHOW CPU DMAn			show DMA channel n

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

P			15		program counter

A			16		A register

B			16		B register

X			16		index register

SC			16		shift count

C			1		carry flag

EXT			1		extend flag

PME			1		previous mode extend flag

EXT_OFF		1		extend off pending flag

DP			1		double precision flag

SS1..4		1		sense switches 1 to 4

ION			1		interrupts enabled

INODEF		1		interrupts not deferred

INTREQ		16		interrupt requests

DEVRDY		16		device ready flags (read only)

DEVENB		16		device interrupt enable flags (read only)

CHREQ			20		DMA/DMC channel requests

DMAAD[0:3]		16		DMA channel current address, channels 1 to 4

DMAWC[0:3]		15		DMA channel word count, channels 1 to 4

DMAEOR[0:3]		1		DMA end of range flag, channels 1 to 4

STOP_INST		1		stop on undefined instruction

STOP_DEV		1		stop on undefined device

INDMAX		8		indirect address limit

PCQ[0:63]		15		PC prior to last JMP, JSB, or interrupt;

most recent PC change first

WRU			8		interrupt character

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

The maximum length for the history is 65,536 entries.

## Programmed I/O Devices

### 316/516-50 Paper Tape Reader (PTR)

The paper tape reader (PTR) reads data from a disk file.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

The paper tape reader can bet set to operate in binary, ASCII, or Unix ASCII mode:

SET PTR BINARY			binary mode

SET PTR ASCII			ASCII mode

SET PTR UASCII			Unix ASCII mode

The mode can also be set by a switch setting in the ATTACH command:

ATT –B PTR <file>			binary mode

ATT –A PTR <file>			ASCII mode

ATT –U PTR <file>			Unix ASCII mode

In ASCII or Unix ASCII mode, all non-zero characters have the high order bit forced on.  In Unix ASCII mode, newline is converted to CR, and LF is inserted as the following character.

The paper tape reader supports the BOOT command.  BOOT PTR copies the absolute binary loader into memory and starts it running.

The paper tape reader implements these registers:

name			size		comments

BUF			8		last data item processed

INTREQ		1		device interrupt request

READY			1		device ready

ENABLE		1		device interrupts enabled

POS			32		position in the input file

TIME			24		time from I/O initiation to interrupt

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

end of file		1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### 316/516-52 Paper Tape Punch (PTP)

The paper tape punch (PTP) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

The paper tape punch can bet set to operate in binary, ASCII, or Unix ASCII mode:

SET PTP BINARY			binary mode

SET PTP ASCII			ASCII mode

SET PTP UASCII			Unix ASCII mode

The mode can also be set by a switch setting in the ATTACH command:

ATT –B PTP <file>			binary mode

ATT –A PTP <file>			ASCII mode

ATT –U PTP <file>			Unix ASCII mode

In ASCII or Unix ASCII mode, all characters are masked to 7b before being written to the output file.  In Unix ASCII mode, LF is converted to newline, and CR is discarded.

The paper tape punch implements these registers:

name			size		comments

BUF			8		last data item processed

INTREQ		1		device interrupt request

READY			1		device ready

ENABLE		1		device interrupts enabled

POWER			1		device powered up

POS			32		position in the output file

TIME			24		time from I/O initiation to interrupt

PWRTIME		24		time from I/O request to power up

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of tape

OS I/O error	x		report error and stop

### 316/516-33 Console Teletype (TTY)

The console Teletype (TTY) consists of four separate units:

TTY0			keyboard

TTY1			printer

TTY2			paper tape reader

TTY3			paper tape punch

The keyboard and printer (TTY0, TTY1) can be set to one of four modes, KSR, 7P, 7B, or 8B:

mode	input characters		output characters

KSR	lower case converted	lower case converted to upper case,

to upper case,		high-order bit cleared,

high-order bit set	non-printing characters suppressed

7P	high-order bit cleared	high-order bit cleared,

non-printing characters suppressed

7B	high-order bit cleared	high-order bit cleared

8B	no changes			no changes

The default mode is KSR.  The Teletype keyboard reads from the console keyboard, and the printer writes to the simulator console window.

The paper tape reader (TTY2) can bet set to operate in binary, ASCII, or Unix ASCII mode:

SET TTY2 BINARY			binary mode

SET TTY2 ASCII			ASCII mode

SET TTY2 UASCII			Unix ASCII mode

The mode can also be set by a switch setting in the ATTACH command:

ATT –B TTY2 <file>		binary mode

ATT –A TTY2 <file>		ASCII mode

ATT –U TTY2 <file>		Unix ASCII mode

In ASCII or Unix ASCII mode, all non-zero characters have the high order bit forced on.  In Unix ASCII mode, newline is converted to CR, and LF is inserted as the following character.

The paper tape reader is started by program output of XON or by the command SET TTY2 START.  The paper tape reader is stopped by reader input of XOFF or by the command SET TTY2 STOP.

The Teletype paper tape punch (TTY3) can bet set to operate in binary, ASCII, or Unix ASCII mode:

SET TTY3 BINARY			binary mode

SET TTY3 ASCII			ASCII mode

SET TTY3 UASCII			Unix ASCII mode

The mode can also be set by a switch setting in the ATTACH command:

ATT –B TTY3 <file>		binary mode

ATT –A TTY3 <file>		ASCII mode

ATT –U TTY3 <file>		Unix ASCII mode

In ASCII or Unix ASCII mode, all characters are masked to 7b before being written to the output file.  In Unix ASCII mode, LF is converted to newline, and CR is discarded.

The Teletype paper tape punch is started by program output of TAPE or by the command SET TTY3 START.  The punch is stopped by program output of XOFF or by the command SET TTY3 STOP.

The TTY implements these registers:

name			size		comments

BUF			8		last data item processed

IN2ND			9		holding buffer, input busy wait; the high

order bit indicates character present

MODE			1		read/write mode

READY			1		device ready flag

BUSY			1		device busy flag

INT			1		device interrupt request

ENABLE		1		device interrupt enabled

KPOS			32		number of keyboard characters input

KTIME			24		keyboard polling interval

KBTIME		24		keyboard busy wait after receive

TPOS			32		number of printer characters output

TTIME			24		time from I/O initiation to interrupt

RPOS			32		current reader character position

PPOS			32		current punch character position

### 316/516-12 Real Time Clock (CLK)

The real time clock (CLK) frequency can be adjusted as follows:

SET CLK 60HZ			set frequency to 60Hz

SET CLK 50HZ			set frequency to 50Hz

The default is 60Hz.

The clock implements these registers:

name			size		comments

INTREQ		1		device interrupt request

READY			1		device ready

ENABLE		1		device interrupts enabled

TIME			24		clock interval

The real-time clock autocalibrates; the clock interval is adjusted up or down so that the clock tracks actual elapsed time.

## 316/516 Line Printer (LPT)

The line printer (LPT) writes data to a disk file.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer can be connected to the IO bus, a DMC channel, or a DMA channel:

SET LPT IOBUS			connect to IO bus

SET LPT DMC=n			connect to DMC channel n (1-16)

SET LPT DMA=n			connect to DMA channel n (1 to 4)

By default, the line printer is connected to the IO bus.

The line printer implements these registers:

name			size		comments

WDPOS			6		word position in current scan

DRPOS			6		drum position

CRPOS			1		carriage position

PRDN			1		print done flag

RDY			1		ready flag

EOR			1		(DMA/DMC) end of range flag

DMA			1		transfer using DMA/DMC

INTREQ		1		device interrupt request

ENABLE		1		device interrupt enable

SVCST			2		service state

SVCCH			2		service channel

BUF			8		buffer

POS			32		position in the output file

XTIME			24		delay between transfers

ETIME			24		delay at end of scan

PTIME			24		delay for shuttle/line advance

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		out of paper

OS I/O error	x		report error and stop

## 4400 Fixed Head Disk (FHD)

Fixed head disk options include the ability to set the number of surfaces to a fixed value between 1 and 16, or to autosize the number of surfaces from the attached file:

SET FHD 1S				one surface (98K)

SET FHD 2S				two platters (196K)

:

SET FHD 16S				sixteen surfaces (1568K)

SET FHD AUTOSIZE			autosized on ATTACH

The default is one surface.

The fixed head disk can be connected to the IO bus, a DMC channel, or a DMA channel:

SET FHD IOBUS			connect to IO bus

SET FHD DMC=n			connect to DMC channel n (1-16)

SET FHD DMA=n			connect to DMA channel n (1 to 4)

By default, the fixed head disk is connected to the IO bus.

The fixed head disk implements these registers:

name			size		comments

CW1			16		control word 1 (read write, surface, track)

CW2			16		control word 2 (character address)

BUF			16		data buffer

BUSY			1		controller busy flag

RDY			1		transfer ready flag

DTE			1		data transfer error flag

ACE			1		access error flag

EOR			1		(DMA/DMC) end of range

DMA			1		transfer using DMA/DMC

CSUM			1		transfer parity checksum

INTREQ		1		device interrupt request

ENABLE		1		device interrupt enable

TIME			24		delay between words

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error	        STOP_IOE		processed as

not attached	1		report error and stop

0		disk not ready

Fixed head disk data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## 4100 7-track Magnetic Tape (MT)

Magnetic tape options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.

The magnetic tape controller can be connected to the IO bus, a DMC channel, or a DMA channel:

SET MT IOBUS			connect to IO bus

SET MT DMC=n			connect to DMC channel n (1-16)

SET MT DMA=n			connect to DMA channel n (1 to 4)

By default, the magnetic tape controller is connected to the IO bus.

The magnetic tape controller implements these registers:

name			size		comments

BUF			16		data buffer

USEL			2		unit select

BUSY			1		controller busy flag

RDY			1		transfer ready flag

ERR			1		error flag

EOF			1		end of file flag

EOR			1		(DMA/DMC) end of range

DMA			1		transfer using DMA/DMC

MDIRQ			1		motion done interrupt request

INTREQ		1		device interrupt request

ENABLE		1		device interrupt enable

DBUF[0:65535]	8		transfer buffer

BPTR			17		transfer buffer pointer

BMAX			17		transfer size (reads)

CTIME			24		start/stop time

XTIME			24		delay between words

POS[0:3]		32		position, units 0 to 3

STOP_IOE		1		stop on I/O error

Error handling is as follows:

error			processed as

not attached	tape not ready; if STOP_IOE, stop

end of file		bad tape

OS I/O error	parity error; if STOP_IOE, stop

## 4623/4651/4720 Disk Packs (DP)

The disk controller can be configured as a 4623, supporting 10-surface disk packs; a 4651, supporting 2-surface disk packs; or a 4720, supporting 20-surface disk packs:

SET DP 4623				controller is 4623

SET DP 4651				controller is 4651

SET DP 4720				controller is 4720

The default is 4651.  All disk packs on the controller must be of the same type.

Individual units can be write enabled or write locked:

SET DPn LOCKED			set unit n write locked

SET DPn WRITEENABLED		set unit n write enabled

Units can be also be set ENABLED or DISABLED.

The disk pack controller can be connected to a DMC channel or a DMA channel; it cannot be connected to the IO bus:

SET DP DMC=n			connect to DMC channel n (1-16)

SET DP DMA=n			connect to DMA channel n (1 to 4)

The disk pack controller supports variable track formatting.  Each track can contain between 1 and 103 records, with a minimum size of 1 word and a maximum size of 1893 words.  Record addresses are unconstrained.  The simulator provides a command to perform a simple, fixed record size format of a new disk:

SET DPn FORMAT=k			format unit n with k words per record

SET -R DPn FORMAT=k		format unit n with k records per track

Record addresses can either be geometric (cylinder/track/sector) or simple sequential starting from 0:

SET DPn FORMAT=k			format with geometric record addresses

SET -S DPn FORMAT=k		format with sequential record addresses

Geometric address have the cylinder number in bits<1:8>, the head number in bits<9:13>, and the sector number in bits <14:16>.

A summary of the current format, and its validity, can be obtained with the command:

SHOW DPn FORMAT			display format of unit n

To accommodate the variable formatting, each track is allocated 2048 words in the data file.  A record consists of a three word header, the data, and a five word trailer:

word 0		record length in words, not including header/trailer

word 1		record address

word 2		number of extension words used (0-4)

word 3		start of data record

word 3+n-1		end of data record

word 3+n..7+n	record trailer: up to four extension words, plus

checksum

A record can "grow" by up to four words without disrupting the track formatting; writing more than four extra words destroys the formatting of the rest of the track and causes a simulator error.

The disk pack controller implements these registers:

name			size		comments

STA			16		status

BUF			16		data buffer

FNC			4		controller function

CW1			16		command word 1

CW2			16		command word 2

CSUM			16		record checksum

BUSY			1		controller busy

RDY			1		transfer ready

EOR			1		(DMA/DMC) end of range

DEFINT		1		seek deferred interrupt pending

INTREQ		1		interrupt request

ENABLE		1		interrupt enable

TBUF[0:2047]	16		track buffer

RPTR			11		pointer to start of record in track buffer

WPTR			11		pointer to current word in record

BCTR			15		bit counter for formatting

STIME			24		seek time, per cylinder

XTIME			24		transfer time, per word

BTIME			24		controller busy time

Error handling is as follows:

error			processed as

not attached	pack off line; if STOP_IOE, stop

end of file		ignored

OS I/O error	data error; if STOP_IOE, stop

# Symbolic Display and Input

The H316/H516 simulator implements symbolic display and input.  Display is controlled by command line switches:

-a			display as ASCII character

-c			display as two packed ASCII characters

-m			display instruction mnemonics

Input parsing is controlled by the first character typed in or by command line switches:

' or -a		ASCII character

" or -c		two packed ASCII characters

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard H316/H516 assembler syntax.  There are six instruction classes: memory reference, I/O, control, shift, skip, and operate.

Memory reference instructions have the format

memref{*} {C/Z} address{,1}

where * signifies indirect, C a current sector reference, Z a sector zero reference, and 1 indexed.  The address is an octal number in the range 0 - 077777; if C or Z is specified, the address is a page offset in the range 0 - 0777.  Normally, C is not needed; the simulator figures out from the address what mode to use.  However, when referencing memory outside the CPU, there is no valid PC, and C must be used to specify current sector addressing.

I/O instructions have the format

io function,device

The function is an octal number in the range 0-17. The device is a symbolic name (e.g., TTY) or an octal number in the range 0-77.

Control and operate instructions consist of a single opcode

opcode

Shift instructions have the format

shift n

where n is an octal number in the range 0-77.

Skip instructions have the format

sub-op sub-op sub-op...

The simulator checks that the combination of sub-opcodes is legal.